Dahua 4/8-Port PoE Switch User’s Manual

V1.0.0

Dahua Technology CO., LTD

Important Safeguards and Warnings

Attention:

Please read the following safeguards and warnings carefully before using the product in order to avoid damages and losses.

Note:

    - Do not expose the device to lampblack, steam or dust. Otherwise it may cause fire or electric shock.

    - Do not install the device at position exposed to sunlight or in high temperature. Temperature rise in

device may cause fire.

    - Do not expose the device to humid environment. Otherwise it may cause fire.

    - The device must be installed on solid and flat surface in order to guarantee safety under load and earthquake. Otherwise, it may cause device to fall off or turnover.

    - Do not place the device on carpet or quilt.

    - Do not block air vent of the device or ventilation around the device. Otherwise, temperature in

device will rise and may cause fire.

    - Do not place any object on the device.

    - Do not disassemble the device without professional instruction.

Warning:

    - Please use battery properly to avoid fire, explosion and other dangers.

    - Please replace used battery with battery of the same type.

    - Do not use power line other than the one specified. Please use it properly. Otherwise, it may cause fire or electric shock.

Special Announcement:

    - This manual is for reference only.

    - All the designs and software here are subject to change without prior written notice.

    - All trademarks and registered trademarks are the properties of their respective owners.

    - If there is any uncertainty or controversy, please refer to the final explanation of us.

    - Please visit our website for more information.

Applicable Models:

The manual can be applied to the following models:

DH-PFS3106-4P-60, DH-PFS3110-8P-96

Table of Contents

1	Product Overview	- 1 -

1.1	Features	- 1 -

1.2	Typical Application	- 1 -

2	Device Structure	- 3 -

2.1	4-Port PoE Switch	- 3 -

2.1.1	Front Panel	- 3 -

2.1.2	Upper Cover	- 5 -

2.1.3	PoE Power Supply	- 6 -

2.2	8-Port PoE Switch	- 7 -

2.2.1	Front Panel	- 7 -

2.2.2	Upper Cover	- 8 -

2.2.3	PoE Power Supply	- 9 -

3	Installation Guide	- 10 -

	Appendix 1 Technical Specification	- 11 -

Product Overview

Features

- Common Features:

  - Two-layer industrial POE switch.

  - Conform IEEE802.3, IEEE802.3u, IEEE802.3ab/z and IEEE802.3X standards.

  - MAC auto study and aging, MAC address list capacity is 8K.

  - All ports self-adapt MDI/MDIX mode.

  - All the10/100M self-adaptive RJ45 ports support IEEE802.3af, IEEE802.3at standard power supply.

  - Industrial wide temperature design.

  - Adopt metal structure.

  - Support DC 53V power supply.

- Individual Features:

  - 4-port PoE switch 1 100/1000M self-adaptive SFP fiber port, 1 10/100/1000M self-adaptive RJ45 port and 4 10/100M self-adaptive RJ 45 ports.

  - 4-port PoE switch with 60W power adapter.

  - 8-port PoE switch supports 1 1000M SFP fiber port, 1 10/100/1000M self-adaptive RJ45 port and 8 10/100M self-adaptive RJ 45 ports.

  - 8-port PoE switch with 96W power adapter.

Typical Application

The typical application of the device is shown in Figure 1- 1

Figure 1- 1

Device Structure

4-Port PoE Switch

Front Panel

The front panel is shown in Figure 2-1.

Figure 2-1

| SN | Name | Note |
| --- | --- | --- |
| 1 | 10/100 Base-T | 4 10/100M self-adaptive PoE power supply ports. |
| 2 | 10/100/1000 Base-T | 10/100/1000M self-adaptive RJ45 port. |
| 3 | 100/1000 Base-X | 100/1000M self-adaptive SFP fiber port. |
| 4 | Link/Act | Fiber port status indicator |
| 5 | PWR | Power indicator |

Sheet 2-1

Upper Cover

The device power port is shown in Figure 2-2, support DC53V power supply.

Figure 2-2

PoE Power Supply

4 10/100M RJ45 ports support IEEE802.3af, IEEE802.3at standard power supply.

Support 4 10/100M RJ45 Ports IEEE802.3af standard power supply simultaneously.

Support 2 10/100M RJ45 ports IEEE802.3at standard power supply simultaneously.

8-Port PoE Switch

Front Panel

Figure 2-3

| SN | Name | Note |
| --- | --- | --- |
| 1 | 10/100 Base-T | 8 10/100M self-adaptive RJ45 ports, which are used for PoE power supply. |
| 2 | 10/100/1000 Base-T | 10/100/1000M self-adaptive RJ45 port. |
| 3 | 1000 Base-X | 1000M self-adaptive SFP fiber port. |
| 4 | Link/Act | Fiber port status indicator. |
| 5 | PWR | Power indicator. |

Sheet 2-2

Upper Cover

The device power port is shown in Figure 2-4, support DC53V power supply.

Figure 2-4

PoE Power Supply

8 10/100M RJ45 ports support IEEE802.3af, IEEE802.3at standard power supply.

Support several RJ45 Ports IEEE802.3af and IEEE802.3at standard power supply simultaneously,

- total power consumption of PoE power supply is less than 93W.

Installation Guide

- PoE switch supports DIN rail mounting. Lay the switch hook on the rail, press the PoE switch to make

- the buckle get into the slide, see Figure 3-1.

- Note:

4-port PoE switch supports the slide width of 28mm.

8-port PoE switch supports the slide width of 38mm.

Figure 3-1

Appendix 1 Technical Specification

| Technical Parameters | DH-PFS3106-4P-60 | DH-PFS3110-8P-96 |
| --- | --- | --- |
| Physical Port |  |  |
| Business Port | 1*100/1000 Base-X、 1*10/100/1000 Base-T、 4*10/100 Base-T（POE power supply） | 1*1000 Base-X、 1*10/100/1000 Base-T、 8*10/100 Base-T（POE power supply） |
| Technical Index |  |  |
| Exchange Capacity | 4.80Gbps | 5.60Gbps |
| Packet Forwarding Rate | 3.57Mpps | 4.17Mpps |
| Exchange Mode | Store & forward | Store & forward |
| MAC Study | MAC auto study, address list capacity 8K | MAC auto study, address list capacity 8K |
| Common Parameters |  |  |
| Lightning Protection Level | Lightning protection level 3 | Lightning protection level 3 |
| Indicator | Power indicator, fiber port status indicator | Power indicator, fiber port status indicator |
| Power | Default DC53V power adapter | Default DC53V power adapter |
| Power Consumption | ≤60W | ≤96W |
| Application Humidity | 10%～90% | 10%～90% |
| High &Low Temperature | ﹣30℃～65℃ | ﹣30℃～65℃ |
| Weight | 480g | 540g |
| Dimension | 150mm×100mm×30mm | 150mm×100mm×42mm |

Note

- This user’s manual is for reference only.

- Slight difference may be found in user interface.

- All the designs and software here are subject to change without prior written notice.

- All trademarks and registered trademarks are the properties of their respective owners.

- If there is any uncertainty or controversy, please refer to the final explanation of us.

- Please visit our website for more information.

Dahua Technology Co., Ltd

Address：No.1199 Bin’an Road, Binjiang District, Hangzhou, China.

Postcode: 310053

Tel: +86-571-87688883

Fax: +86-571-87688815

Email:overseas@dahuatech.com

Website: www.dahuatech.com